推箱子的串口实现

第一步：创建工程

“连接”→“LPUART1”→模式“异步”

图7.4 Pinout & Configuration标签页面

如图所示，设置完成之后，他将默认选择PC0和PC1管脚。

然后，将其分别映射在PA2与PA3管脚。

图7.6 重新分配完的串口引脚分配(视图90︒旋转)

图7.5 默认的串口引脚分配

接下来配置时钟：在“时钟配置”选项卡下。

以最大时钟64MHz频率运行微控制器，即HCLK=64；届时，软件将自动为LPUART1配置为64Mhz。

图7.7 Clock Configuration界面

接下来配置LPUART1：

波特率11520			字长8

图7.8 设置输出工程的参数

奇偶校检无			停止位“1”

第二步：生成代码

第三步：编写函数

①串口的重定向：

#include "stdio.h"

#include "string.h"

#define unit8_t unsigned char

#define PUTCHAR_PROTOTYPE int fputc(int ch,FILE *f)  //函数定义

#define GETCHAR_PROTOTYPE int fgetc(FILE *f)

#define BACKSPACE_PROTOTYPE int _backspace(FILE *f)

PUTCHAR_PROTOTYPE	//重定向 fputc()函数

{

HAL_UART_Transmit(&hlpuart1,(unit8_t*) &ch,1,0xFFFF);  //调用串口发送函数

return ch;		//返回发送的字符

}

GETCHAR_PROTOTYPE //重定向 fgetc()函数

{

uint8_t value; //定义无符号字符型变量 value

while((LPUART1->ISR & 0x00000020)==0){} //判断串口是否接收到字符

value=(uint8_t)LPUART1->RDR; //读取串口接收到的字符

HAL_UART_Transmit(&hlpuart1,(uint8_t *)&value,1,0x1000); //回显接收到的字符

return value; //返回接收到的值 value

}

BACKSPACE_PROTOTYPE //重定向__backspace 函数

{

return 0;

}

②推箱子运行逻辑相关函数定义与内容

void level_pushbox(int k);   //关卡地图初始化

void input_pushbox(int ch);  //输入按键判断逻辑

void supplement_pushbox();    //目标点补充函数

void success_pushbox();       //通关之后运行

void output_pushbox();        //推箱子显示输出函数

void level_pushbox(int GK)  //关卡地图初始化

{

switch(GK)

{

case 1:

{

strcpy(a[0],"  ###                                           ");

strcpy(a[1],"   #*#                                           ");

strcpy(a[2],"   # #                                           ");

strcpy(a[3],"####O######                                      ");

strcpy(a[4],"#*  OM O *#                                      ");

strcpy(a[5],"#####O#####                                      ");

strcpy(a[6],"    # #                                          ");

strcpy(a[7],"    #*#                                          ");

strcpy(a[8],"    ###                                          ");

strcpy(a[9],"                                                 ");

strcpy(a[10],"关卡 1/5                                        ");

strcpy(a[11],"                                                ");

strcpy(a[12],"                                                ");

strcpy(a[13],"                                                ");

strcpy(a[14],"                                                ");

Player_Coordinates_Y=4,Player_Coordinates_X=5;

Box1_Destination_Y=1,Box1_Destination_X=4;

Box2_Destination_Y=4,Box2_Destination_X=1;

Box3_Destination_Y=4,Box3_Destination_X=9;

Box4_Destination_Y=7,Box4_Destination_X=5;

}

break;

case 2:

{

strcpy(a[0],"######                                           ");

strcpy(a[1],"#*   #                                           ");

strcpy(a[2],"###  #                                           ");

strcpy(a[3],"#  O ######                                      ");

strcpy(a[4],"#*  OM O *#                                      ");

strcpy(a[5],"#####O#####                                      ");

strcpy(a[6],"    # #                                          ");

strcpy(a[7],"    #*#                                          ");

strcpy(a[8],"    ###                                          ");

strcpy(a[9],"                                                 ");

strcpy(a[10],"关卡 2/5                                        ");

strcpy(a[11],"                                                ");

strcpy(a[12],"                                                ");

strcpy(a[13],"                                                ");

strcpy(a[14],"                                                ");

Player_Coordinates_Y=4,Player_Coordinates_X=5;

Box1_Destination_Y=1,Box1_Destination_X=1;

Box2_Destination_Y=4,Box2_Destination_X=1;

Box3_Destination_Y=4,Box3_Destination_X=9;

Box4_Destination_Y=7,Box4_Destination_X=5;

}

break;

case 3:

{

strcpy(a[0],"  ####                                           ");

strcpy(a[1],"  #  #                                           ");

strcpy(a[2],"  #  #                                           ");

strcpy(a[3],"  #M #                                           ");

strcpy(a[4],"### ######                                       ");

strcpy(a[5],"#   O  O*#                                       ");

strcpy(a[6],"# O*   ###                                       ");

strcpy(a[7],"#####* O*#                                       ");

strcpy(a[8],"    ######                                       ");

strcpy(a[9],"                                                 ");

strcpy(a[10],"关卡 3/5                                        ");

strcpy(a[11],"                                                ");

strcpy(a[12],"                                                ");

strcpy(a[13],"                                                ");

strcpy(a[14],"                                                ");

Player_Coordinates_Y=3,Player_Coordinates_X=3;

Box1_Destination_Y=6,Box1_Destination_X=3;

Box2_Destination_Y=5,Box2_Destination_X=8;

Box3_Destination_Y=7,Box3_Destination_X=5;

Box4_Destination_Y=7,Box4_Destination_X=8;

}

break;

case 4:

{

strcpy(a[0]," ########                                        ");

strcpy(a[1]," #     ###                                       ");

strcpy(a[2],"##O###   #                                       ");

strcpy(a[3],"#M  O  O #                                       ");

strcpy(a[4],"# **# O ##                                       ");

strcpy(a[5],"##**#   #                                        ");

strcpy(a[6]," ########                                        ");

strcpy(a[7],"                                                 ");

strcpy(a[8],"                                                 ");

strcpy(a[9],"                                                 ");

strcpy(a[10],"关卡 4/5                                        ");

strcpy(a[11],"                                                ");

strcpy(a[12],"                                                ");

strcpy(a[13],"                                                ");

strcpy(a[14],"                                                ");

Player_Coordinates_Y=3,Player_Coordinates_X=1;

Box1_Destination_Y=4,Box1_Destination_X=2;

Box2_Destination_Y=4,Box2_Destination_X=3;

Box3_Destination_Y=5,Box3_Destination_X=2;

Box4_Destination_Y=5,Box4_Destination_X=3;

}

break;

case 5:

{

strcpy(a[0],"  ####                                           ");

strcpy(a[1],"  #**#                                           ");

strcpy(a[2]," ## *##                                          ");

strcpy(a[3]," #  O*#                                          ");

strcpy(a[4],"## O  ##                                         ");

strcpy(a[5],"#  #OO #                                         ");

strcpy(a[6],"#  M   #                                         ");

strcpy(a[7],"########                                         ");

strcpy(a[8],"                                                 ");

strcpy(a[9],"                                                 ");

strcpy(a[10],"关卡 5/5                                        ");

strcpy(a[11],"                                                ");

strcpy(a[12],"                                                ");

strcpy(a[13],"                                                ");

strcpy(a[14],"                                                ");

Player_Coordinates_Y=6,Player_Coordinates_X=3;

Box1_Destination_Y=1,Box1_Destination_X=3;

Box2_Destination_Y=1,Box2_Destination_X=4;

Box3_Destination_Y=2,Box3_Destination_X=4;

Box4_Destination_Y=3,Box4_Destination_X=5;

}

break;

case 0:

{

strcpy(a[0],"             通关攻略                           ");

strcpy(a[1],"关卡 1/5 :                                       ");

strcpy(a[2],"  SS WW DDD AAAAAA DD WW                         ");

strcpy(a[3],"关卡 2/5 :                                       ");

strcpy(a[4],"  SS WW DDD AAAAAA DWW DWAA                      ");

strcpy(a[5],"关卡 3/5 :                                       ");

strcpy(a[6],"  SSSDDDWDA SSDAWWAASAW AASDDDWDS AAWWWD WWASSSS ");

strcpy(a[7],"关卡 4/5 :                                       ");

strcpy(a[8],"  DDDD SSDDWWA WWAAAA SSASDWDS WAWWDDDD SSAAADDD ");

strcpy(a[9],"  WWAAAA SSASD WDDDDD SASAW DWAAA DDDWW AAAA SSS ");

strcpy(a[10],"  WDDDDDD WAS AAAAA WWDDDD SDS AAAA DDDWW AAAASS");

strcpy(a[11],"关卡 5/5 :                                      ");

strcpy(a[12],"  AWW DDWW SSAASS DDWWW SSSDD WASAAA WWWDDD SAAA");

strcpy(a[13],"  SSDD WW DWA SAWW SSASS DDDWW                  ");

strcpy(a[14],"                                                ");

}

break;

}

}

void input_pushbox(int ch)     //输入按键判断逻辑

{

switch(ch)  //按键判断逻辑

{

case 8:

if(a[Player_Coordinates_Y-1][Player_Coordinates_X]!='#')  //上边不是墙

{

if(a[Player_Coordinates_Y-1][Player_Coordinates_X]!='O'&&a[Player_Coordinates_Y-1][Player_Coordinates_X]!='@') //上边不是箱子

{

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';   //当前位置人物消失

Player_Coordinates_Y--;                              //人物上移

a[Player_Coordinates_Y][Player_Coordinates_X]='M';   //人物出现

}

else if

(a[Player_Coordinates_Y-2][Player_Coordinates_X]!='#'&&a[Player_Coordinates_Y-2][Player_Coordinates_X]!='@'&&a[Player_Coordinates_Y-2][Player_Coordinates_X]!='O')

{    //上边是箱子  上上不是墙也不是箱子

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';   //同

Player_Coordinates_Y--;

a[Player_Coordinates_Y][Player_Coordinates_X]='M';   //上

a[Player_Coordinates_Y-1][Player_Coordinates_X]='O'; //箱子出现在人上方  推箱操作成功

if(((Player_Coordinates_Y-1==Box1_Destination_Y)&&(Player_Coordinates_X==Box1_Destination_X))

||((Player_Coordinates_Y-1==Box2_Destination_Y)&&(Player_Coordinates_X==Box2_Destination_X))				    ||((Player_Coordinates_Y-1==Box3_Destination_Y)&&(Player_Coordinates_X==Box3_Destination_X))

||((Player_Coordinates_Y-1==Box4_Destination_Y)&&(Player_Coordinates_X==Box4_Destination_X)))

a[Player_Coordinates_Y-1][Player_Coordinates_X]='@';//判断箱子是否在目标点

}

}

break;

case 4:

if(a[Player_Coordinates_Y][Player_Coordinates_X-1]!='#')

{

if(a[Player_Coordinates_Y][Player_Coordinates_X-1]!='O'&&a[Player_Coordinates_Y][Player_Coordinates_X-1]!='@')

{

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';

Player_Coordinates_X--;

a[Player_Coordinates_Y][Player_Coordinates_X]='M';

}

else if(a[Player_Coordinates_Y][Player_Coordinates_X-2]!='#'&&a[Player_Coordinates_Y][Player_Coordinates_X-2]!='@'&&a[Player_Coordinates_Y][Player_Coordinates_X-2]!='O')

{

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';

Player_Coordinates_X--;

a[Player_Coordinates_Y][Player_Coordinates_X]='M';

a[Player_Coordinates_Y][Player_Coordinates_X-1]='O';

if(((Player_Coordinates_Y==Box1_Destination_Y)&&(Player_Coordinates_X-1==Box1_Destination_X))

||((Player_Coordinates_Y==Box2_Destination_Y)&&(Player_Coordinates_X-1==Box2_Destination_X))

||((Player_Coordinates_Y==Box3_Destination_Y)&&(Player_Coordinates_X-1==Box3_Destination_X))

||((Player_Coordinates_Y==Box4_Destination_Y)&&(Player_Coordinates_X-1==Box4_Destination_X)))

a[Player_Coordinates_Y][Player_Coordinates_X-1]='@';

}

}

break;

case 2:

if(a[Player_Coordinates_Y+1][Player_Coordinates_X]!='#')

{

if(a[Player_Coordinates_Y+1][Player_Coordinates_X]!='O'&&a[Player_Coordinates_Y+1][Player_Coordinates_X]!='@')

{

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';

Player_Coordinates_Y++;

a[Player_Coordinates_Y][Player_Coordinates_X]='M';

}			    else if(a[Player_Coordinates_Y+2][Player_Coordinates_X]!='#'&&a[Player_Coordinates_Y+2][Player_Coordinates_X]!='@'&&a[Player_Coordinates_Y+2][Player_Coordinates_X]!='O')

{

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';

Player_Coordinates_Y++;

a[Player_Coordinates_Y][Player_Coordinates_X]='M';

a[Player_Coordinates_Y+1][Player_Coordinates_X]='O';

if(((Player_Coordinates_Y+1==Box1_Destination_Y)&&(Player_Coordinates_X==Box1_Destination_X))

||((Player_Coordinates_Y+1==Box2_Destination_Y)&&(Player_Coordinates_X==Box2_Destination_X))

||((Player_Coordinates_Y+1==Box3_Destination_Y)&&(Player_Coordinates_X==Box3_Destination_X))

||((Player_Coordinates_Y+1==Box4_Destination_Y)&&(Player_Coordinates_X==Box4_Destination_X)))

a[Player_Coordinates_Y+1][Player_Coordinates_X]='@';

}

}

break;

case 6:

if(a[Player_Coordinates_Y][Player_Coordinates_X+1]!='#')

{		    if(a[Player_Coordinates_Y][Player_Coordinates_X+1]!='O'&&a[Player_Coordinates_Y][Player_Coordinates_X+1]!='@')

{

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';

Player_Coordinates_X++;

a[Player_Coordinates_Y][Player_Coordinates_X]='M';

}

else If

(a[Player_Coordinates_Y][Player_Coordinates_X+2]!='#'&&a[Player_Coordinates_Y][Player_Coordinates_X+2]!='@'&&a[Player_Coordinates_Y][Player_Coordinates_X+2]!='O')

{

a[Player_Coordinates_Y][Player_Coordinates_X]=' ';

Player_Coordinates_X++;

a[Player_Coordinates_Y][Player_Coordinates_X]='M';

a[Player_Coordinates_Y][Player_Coordinates_X+1]='O';				   if(((Player_Coordinates_Y==Box1_Destination_Y)&&(Player_Coordinates_X+1==Box1_Destination_X))

||((Player_Coordinates_Y==Box2_Destination_Y)&&(Player_Coordinates_X+1==Box2_Destination_X))

||((Player_Coordinates_Y==Box3_Destination_Y)&&(Player_Coordinates_X+1==Box3_Destination_X))

||((Player_Coordinates_Y==Box4_Destination_Y)&&(Player_Coordinates_X+1==Box4_Destination_X)))

a[Player_Coordinates_Y][Player_Coordinates_X+1]='@';

}

}

break;

default:

printf("\r\n  违规操作！！！ \r\n");;   //按下其他按键不响应

break;

}

}

void supplement_pushbox()  //目标点补充函数

{

if(a[Box1_Destination_Y][Box1_Destination_X]==' ')

a[Box1_Destination_Y][Box1_Destination_X]='*';

if(a[Box2_Destination_Y][Box2_Destination_X]==' ')

a[Box2_Destination_Y][Box2_Destination_X]='*';

if(a[Box3_Destination_Y][Box3_Destination_X]==' ')

a[Box3_Destination_Y][Box3_Destination_X]='*';

if(a[Box4_Destination_Y][Box4_Destination_X]==' ')

a[Box4_Destination_Y][Box4_Destination_X]='*';

}

void success_pushbox()  //通关之后运行

{

printf("\r\n！！！恭喜通关！！！\r\n");

printf("\r\n 密码是 19991015 \r\n");

for (float y = 1.3f; y > -1.0f; y -= 0.1f)  //画爱心图

{

for (float x = -1.5f; x < 1.5f; x += 0.05f)

{

float a = x * x + y * y - 1;

putchar(a * a * a - x * x * y * y * y <= 0.0f ? '*' : ' ');

}

printf("\r\n");

}

}

void output_pushbox()  //推箱子显示输出函数

{

int i;

int j;

printf("\r\n");

for(i=0;i<15;i++)

{

for(j=0;j<50;j++)

{

printf("%c",a[i][j]);

}

HAL_Delay(20); //必须有延时

printf("\r\n");

}

}

第四步：添加主函数

①相关初始化

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_LPUART1_UART_Init();

②判断是否显示通关攻略

printf("\r\n 请输入密码查看通关攻略: \r\n");

scanf("%d",&PSWD);

HAL_Delay(100);

printf("\r\n Receive PSWD=%d",PSWD);

if(PSWD==19991015)   //判断密码是否正确

{

level_pushbox(0);

output_pushbox();

}

else

printf("\r\n 密码错误 \r\n");

③判断是否解锁全部关卡

printf("\r\n 请输入密码解锁所有关卡: \r\n");

scanf("%d",&PSWD);

HAL_Delay(100);

printf("\r\n Receive PSWD=%d",PSWD);

if(PSWD==19991015)   //判断密码是否正确

{

p=5;

printf("\r\n 密码正确，所有关卡已解锁！！！ \r\n");

}

else

printf("\r\n 密码错误，解锁失败。 \r\n");

④判断是否预览通关画面

printf("\r\n 请输入密码预览通关结果: \r\n");

scanf("%d",&PSWD);

HAL_Delay(100);

printf("\r\n Receive PSWD=%d",PSWD);

if(PSWD==19991015)   //判断密码是否正确

success_pushbox();

else

printf("\r\n 密码错误 \r\n");

⑤关卡选择部分

do

{

printf("请选择关卡(1-5): \r\n");

scanf("%d",&GK);

HAL_Delay(50);

printf("\r\n");

HAL_Delay(50);

if (GK<1||GK>5)

printf("关卡选择错误 \r\n");

else if(GK>p)

printf("禁止白嫖 \r\n");

}while(GK<1||GK>5||GK>p);

⑥游戏运行过程

while(GK>=1&&GK<=5) //游戏执行

{

level_pushbox(GK);  //关卡初始化函数

output_pushbox();  //初始状态显示

while(1)  //运行模块

{

int ch;

printf("请执行操作：");

scanf("%d",&ch);

input_pushbox(ch);     //输入按键判断逻辑

supplement_pushbox();  //目标点补充函数

output_pushbox(); //状态显示刷新

if(a[Box1_Destination_Y][Box1_Destination_X]=='@'&&a[Box2_Destination_Y][Box2_Destination_X]=='@' &&a[Box3_Destination_Y][Box3_Destination_X]=='@'&&

a[Box4_Destination_Y][Box4_Destination_X]=='@')

{

GK++;

if(GK>=p) p=GK; //选关限制----即检查点

HAL_Delay(150);

break; //确认获胜则跳出while(1)循环

}

}

}

⑦通关之后的奖励机制

success_pushbox();//通关之后运行

第四步：编译下载程序

第五步：打开串口助手运行程序

①串口设：选择合适串口（在设备管理器中查看端口）；波特率“115200”；校检位“无校检”；停止位“1位”。

②点击打开串口。

③按下开发板的“复位”按键。

④根据提示向串口发送数据即可。

第六步：结果分享

第七步：问题及其解决总结

①跳转函数  longjmp不能用——用do-while语句和if组合将其代替。

②不能远距离跳跃，例如子函数某个位置跳转到主函数某个位置——用顺序结构搭配少许循环、分支指令代替。适当改变其功能。

③重定位后的字符输入有问题，容易卡死——方向键的实现用数字“8462”代替字符“WASD”。

④原密码用字符串来判断——现在用double类型数字代替[优点：代码更简单/缺点：密码不能包含数字以外其他字符]

⑤误输入字符卡死——输出提示“违规操作”，提示用户“非程序BUG”按下复位键。

⑥输入输出不理想，赋值之后无更改——在变量定义前加关键字“volatile”，它是一种类型修饰符，用它声明的类型变量表示可以被某些编译器未知的因素更改，比如：操作系统、硬件或者其它线程等。

⑦【注意】数字输入的时候，发送完一个数字，必须以回车键结束。

第八步：展望

字符输入问题的解决：回想之前做过的单片机STC8A8K64S4A12串口通信中字符输入是没问题的。仔细回顾，并移植到ARM即可。